Fire Preparation Checklist

I noticed that when a fire started a few hundred feet from our house several summers ago that the first thing to go was my brain.

This is an update of the list I use to remind myself what makes sense to do depending on how close fire is. It includes input from Jan R and S, Rocky, and Wes. If you have more suggestions or changes let me know and I'll add them to the next version

—Art Ludwig            oasisdesign.net

# Fire a day or more away

• Agree on place to meet family members, land line phone number to use as message board for leaving and checking messages

• Take insurance policy, photos of stuff, computer back up, personal valuables to a secure location

• Rake leaves off roof, out of gutters

• Rake leaf litter away from house at least 10 feet

• Don't water buildings or plant leaves yet.

• Use water efficiently so water tanks stay near full

• When tanks are full, store additional water in the soil under plants around buildings. Over the next few days the plants will pump it from the soil into their bodies so they won't  burn as fast or as hot.

• Fill private storage

• Check locations of fire standpipes, hose, connections, gas shutoff, etc.

•Get pet carriers ready

• Park facing out, pack car(s) with stuff to take.

• Practice fire drill with your family and neighbors

# Fire hours away, or to do before you leave

• Shut windows

• Turn off gas line to house

• Turn off water line to house—(especially if it has interior fire sprinklers--the sprinklers inside won't keep everything from burning if the house catches from outside, but they will drain the water tanks)

• Lay out fire hoses

• Block gutter drains at the top and fill them with water

• Foam houses

• Take your pets (and kids) and Leave

# Fire an hour away

• Hose down roof, eaves, walls, and nearby plants

• Turn off water line to house

• Take your pets and Leave

# Fire minutes away

• Leave if you're sure you've got a clear shot, —or—

• Take shelter best shelters are Barrymore's fire bunker, Lee's house downstairs, inside Art's cistern. Smaller shelters may require oxygen.

# Fire Years Away

• Build a structure on your property that can't burn

• Build a fire shelter for stuff and people